Phase Design Comparison: Performance vs Permitted Right Actuated

Color: % change vs Permitted Right Actuated  |  Blue = improvement  |  Red = regression
* p < 0.05   ** p < 0.01  (Welch t-test + Mann-Whitney)

| Low Pedestrian Demand |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Permitted Right |  |  | Protected Right |  |  | LPI |  |  | Delayed Turn |  |  | Exclusive |
| Category | Metric | Actuated | SVCC | M2SVCC | Actuated | SVCC | M2SVCC | Actuated | SVCC | M2SVCC | Actuated | SVCC | M2SVCC | M2SVCC |
| Sustainability Sustainability | Fuel (mg) | 91.12 | 66.48 | 62.30** (-31.6%) | 98.36 | 79.82 | 76.29** (-16.3%) | 94.37 | 74.17 | 68.84** (-24.5%) | 95.86 | 75.58 | 71.65** (-21.4%) | 68.24** (-25.1%) |
|  | Energy (Kw.s) | 207.68 | 201.04 | 193.48** (-6.8%) | 224.13 | 215.87 | 212.92* (+2.5%) | 210.94 | 205.22 | 196.92** (-5.2%) | 226.29 | 214.83 | 213.20 (+2.7%) | 199.83** (-3.8%) |
| Veh Mobility Veh Mobility Veh Mobility | Veh Wait (s) | 28.14 | 30.17 | 23.47** (-16.6%) | 38.68 | 39.77 | 34.01** (+20.8%) | 31.79 | 38.90 | 29.76 (+5.7%) | 36.02 | 35.15 | 26.79 (-4.8%) | 25.74** (-8.5%) |
|  | Veh Time Loss (s) | 43.95 | 45.05 | 37.54** (-14.6%) | 56.33 | 60.01 | 54.27** (+23.5%) | 47.68 | 55.33 | 45.27 (+3.0%) | 53.42 | 53.25 | 42.20 (-4.0%) | 41.53* (-5.5%) |
|  | Veh Queue (m) | 13.83 | 16.53 | 11.88** (-14.1%) | 21.49 | 19.73 | 17.51** (+26.6%) | 15.35 | 17.77 | 13.54 (-2.1%) | 22.52 | 19.74 | 15.33* (+10.8%) | 12.69** (-8.3%) |
| Bike Mobility Bike Mobility Bike Mobility | Bike Wait (s) | 27.02 | 20.05 | 15.94** (-41.0%) | 34.51 | 32.06 | 21.51** (-20.4%) | 29.16 | 25.86 | 17.01** (-37.1%) | 34.66 | 25.66 | 21.15* (-21.7%) | 24.21 (-10.4%) |
|  | Bike Time Loss (s) | 36.56 | 28.39 | 24.09** (-34.1%) | 43.68 | 40.92 | 30.02** (-17.9%) | 39.15 | 34.75 | 25.37** (-30.6%) | 44.45 | 35.62 | 29.35* (-19.7%) | 35.04 (-4.1%) |
|  | Bike Queue (m) | 2.86 | 2.62 | 2.55** (-10.9%) | 3.39 | 3.00 | 2.96 (+3.4%) | 3.05 | 2.72 | 2.69* (-6.0%) | 3.47 | 3.18 | 2.77 (-3.2%) | 2.79 (-2.5%) |
| Ped Mobility | Ped Time Loss (s) | 43.77 | 41.20 | 31.91** (-27.1%) | 49.67 | 52.92 | 35.44** (-19.0%) | 45.40 | 43.04 | 33.13** (-24.3%) | 50.68 | 41.21 | 33.27** (-24.0%) | 39.22* (-10.4%) |
| Safety Safety Safety | Veh-Ped (#) | 61.50 | 63.50 | 54.60 (-11.2%) | 3.80 | 1.67 | 2.14** (-96.5%) | 52.90 | 51.10 | 55.00 (-10.6%) | 22.90 | 46.70 | 42.00* (-31.7%) | 2.00** (-96.7%) |
|  | Veh-Bike (#) | 98.40 | 67.50 | 61.70** (-37.3%) | 65.00 | 45.60 | 37.50** (-61.9%) | 94.50 | 75.80 | 67.10** (-31.8%) | 70.50 | 62.10 | 46.67** (-52.6%) | 45.50** (-53.8%) |
|  | Veh-Veh (#) | 16.11 | 16.00 | 16.60 (+3.0%) | 8.20 | 9.83 | 8.71** (-45.9%) | 13.90 | 15.00 | 12.80 (-20.6%) | 12.20 | 12.12 | 9.00* (-44.1%) | 11.00** (-31.7%) |
| Med Pedestrian Demand |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Permitted Right |  |  | Protected Right |  |  | LPI |  |  | Delayed Turn |  |  | Exclusive |
| Category | Metric | Actuated | SVCC | M2SVCC | Actuated | SVCC | M2SVCC | Actuated | SVCC | M2SVCC | Actuated | SVCC | M2SVCC | M2SVCC |
| Sustainability Sustainability | Fuel (mg) | 95.48 | 70.84 | 68.35** (-28.4%) | 103.66 | 84.90 | 81.67** (-14.5%) | 98.79 | 78.20 | 72.74** (-23.8%) | 98.64 | 80.49 | 78.70** (-17.6%) | 95.29 (-0.2%) |
|  | Energy (Kw.s) | 180.58 | 184.34 | 195.91** (+8.5%) | 210.73 | 203.06 | 200.47** (+11.0%) | 206.50 | 200.72 | 198.45** (+9.9%) | 212.16 | 199.36 | 198.71** (+10.0%) | 208.64** (+15.5%) |
| Veh Mobility Veh Mobility Veh Mobility | Veh Wait (s) | 35.61 | 36.46 | 32.31 (-9.3%) | 45.92 | 45.36 | 41.98* (+17.9%) | 39.71 | 44.18 | 38.96 (+9.4%) | 39.78 | 43.41 | 37.85 (+6.3%) | 57.44** (+61.3%) |
|  | Veh Time Loss (s) | 55.22 | 53.59 | 48.33* (-12.5%) | 67.02 | 69.04 | 64.62** (+17.0%) | 59.10 | 63.84 | 56.77 (+2.8%) | 60.17 | 62.91 | 57.85 (+4.8%) | 81.65** (+47.9%) |
|  | Veh Queue (m) | 15.98 | 15.27 | 13.93* (-12.8%) | 23.30 | 21.22 | 19.71** (+23.4%) | 17.29 | 18.03 | 15.55 (-2.7%) | 24.60 | 21.04 | 19.82** (+24.1%) | 21.34** (+33.6%) |
| Bike Mobility Bike Mobility Bike Mobility | Bike Wait (s) | 29.43 | 25.53 | 20.19** (-31.4%) | 37.01 | 31.44 | 27.28 (-7.3%) | 33.89 | 27.24 | 22.02** (-25.2%) | 34.66 | 28.65 | 21.54** (-26.8%) | 49.53** (+68.3%) |
|  | Bike Time Loss (s) | 40.55 | 35.66 | 30.47** (-24.9%) | 47.31 | 41.35 | 36.77 (-9.3%) | 44.99 | 36.68 | 31.58** (-22.1%) | 45.10 | 38.63 | 30.94** (-23.7%) | 65.75** (+62.1%) |
|  | Bike Queue (m) | 3.41 | 3.04 | 2.98** (-12.7%) | 4.79 | 3.88 | 3.50 (+2.6%) | 3.67 | 3.22 | 3.04** (-10.9%) | 4.49 | 3.86 | 3.36 (-1.5%) | 4.45* (+30.4%) |
| Ped Mobility | Ped Time Loss (s) | 50.11 | 41.68 | 39.89** (-20.4%) | 53.82 | 49.42 | 45.90** (-8.4%) | 51.64 | 43.28 | 38.84** (-22.5%) | 53.91 | 48.25 | 41.05** (-18.1%) | 35.77** (-28.6%) |
| Safety Safety Safety | Veh-Ped (#) | 202.44 | 236.75 | 197.38 (-2.5%) | 10.64 | 3.20 | 5.45** (-97.3%) | 180.22 | 211.00 | 196.83 (-2.8%) | 81.27 | 162.18 | 180.67 (-10.8%) | 3.00** (-98.5%) |
|  | Veh-Bike (#) | 213.56 | 174.25 | 149.75** (-29.9%) | 146.00 | 125.91 | 107.90** (-49.5%) | 202.91 | 177.00 | 150.40** (-29.6%) | 154.50 | 148.00 | 124.86** (-41.5%) | 175.33 (-17.9%) |
|  | Veh-Veh (#) | 35.62 | 34.00 | 35.78 (+0.4%) | 16.91 | 19.36 | 18.09** (-49.2%) | 28.50 | 36.11 | 33.25* (-6.7%) | 21.91 | 31.64 | 30.43 (-14.6%) | 33.33 (-6.4%) |
| High Pedestrian Demand |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Permitted Right |  |  | Protected Right |  |  | LPI |  |  | Delayed Turn |  |  | Exclusive |
| Category | Metric | Actuated | SVCC | M2SVCC | Actuated | SVCC | M2SVCC | Actuated | SVCC | M2SVCC | Actuated | SVCC | M2SVCC | M2SVCC |
| Sustainability Sustainability | Fuel (mg) | 110.06 | 88.85 | 82.58** (-25.0%) | 107.11 | 90.50 | 88.50** (-19.6%) | 114.18 | 101.00 | 95.75** (-13.0%) | 102.46 | 92.46 | 87.18** (-20.8%) | 110.21 (+0.1%) |
|  | Energy (Kw.s) | 197.13 | 203.19 | 194.55 (-1.3%) | 203.60 | 198.25 | 196.84 (-0.1%) | 197.27 | 206.60 | 204.64* (+3.8%) | 205.47 | 194.13 | 187.59** (-4.8%) | 199.00 (+0.9%) |
| Veh Mobility Veh Mobility Veh Mobility | Veh Wait (s) | 52.62 | 59.85 | 52.45 (-0.3%) | 51.45 | 52.66 | 51.00 (-3.1%) | 57.80 | 74.13 | 66.29** (+26.0%) | 45.81 | 56.19 | 50.68 (-3.7%) | 76.89** (+46.1%) |
|  | Veh Time Loss (s) | 79.27 | 81.56 | 72.42 (-8.6%) | 75.09 | 77.71 | 76.68 (-3.3%) | 84.58 | 98.50 | 89.02** (+12.3%) | 69.01 | 81.81 | 73.44 (-7.4%) | 110.52** (+39.4%) |
|  | Veh Queue (m) | 21.12 | 20.52 | 18.33* (-13.2%) | 24.89 | 23.43 | 22.03 (+4.3%) | 22.48 | 22.78 | 22.70 (+7.5%) | 24.64 | 24.70 | 22.58 (+6.9%) | 27.01** (+27.9%) |
| Bike Mobility Bike Mobility Bike Mobility | Bike Wait (s) | 40.97 | 37.90 | 24.73** (-39.6%) | 42.03 | 37.07 | 31.52** (-23.1%) | 44.78 | 38.63 | 36.43** (-11.1%) | 42.19 | 39.32 | 29.18** (-28.8%) | 106.67** (+160.4%) |
|  | Bike Time Loss (s) | 56.35 | 51.28 | 36.23** (-35.7%) | 54.88 | 48.69 | 43.06** (-23.6%) | 60.46 | 51.18 | 49.50** (-12.2%) | 55.45 | 52.21 | 40.25** (-28.6%) | 151.77** (+169.3%) |
|  | Bike Queue (m) | 4.22 | 5.08 | 3.60** (-14.7%) | 6.07 | 4.43 | 4.61 (+9.3%) | 4.39 | 4.21 | 4.99 (+18.3%) | 6.29 | 5.47 | 5.06** (+20.0%) | 13.26** (+214.4%) |
| Ped Mobility | Ped Time Loss (s) | 58.17 | 64.40 | 58.47 (+0.5%) | 60.81 | 56.83 | 52.85** (-9.2%) | 60.02 | 68.25 | 54.45 (-6.4%) | 61.01 | 62.13 | 52.08** (-10.5%) | 45.44** (-21.9%) |
| Safety Safety Safety | Veh-Ped (#) | 280.40 | 275.00 | 309.00 (+10.2%) | 16.62 | 6.00 | 7.20** (-97.4%) | 272.62 | 257.00 | 269.75 (-3.8%) | 130.73 | 248.38 | 214.75** (-23.4%) | 1.20** (-99.6%) |
|  | Veh-Bike (#) | 392.20 | 305.00 | 276.00** (-29.6%) | 257.00 | 216.33 | 203.80** (-48.0%) | 381.00 | 336.50 | 327.25** (-16.6%) | 263.00 | 268.88 | 229.25** (-41.5%) | 313.50** (-20.1%) |
|  | Veh-Veh (#) | 50.20 | 53.00 | 55.00 (+9.6%) | 22.70 | 26.75 | 27.17** (-45.9%) | 42.91 | 50.50 | 60.75 (+21.0%) | 27.36 | 47.62 | 35.00** (-30.3%) | 62.20 (+23.9%) |